Placards To Be Installed

[image: image1.png]










NOTE

Placards are examples of types to be fitted, they will have to be tailor made to suit aircraft e.g. empty weight CG position etc.  Can be printed out and laminated or done with a dymo type labelling machine.

INSTRUMENT PANEL

The instrument panel must have labels to identify each switch, fuse etc.  Switches must always be down for off and labelled ON & OFF.

Each Gauge must have it’s operating ranges shown and maximums or minimums in clear figures above each gauge. 

E.g. Oil Temp Gauge, should show min temp before full throttle and max oil temp.  Also a label to be fixed above the gauge stating Max 118°c.

ASI’s should have a RED radial line at the VNE and also a label above the gauge stating VNE 125mph.
Fuel Placard located inside filler flap





OIL SPEC





AeroShell or similar Brand


W100 Normal use


W80 Cold weather


also 


Aero Oil W Multigrade 15W50


Other oils to MIL-L-22851C





Capacity 2 litres max


No Cooler fitted


2.3 litres with Cooler





FUEL Capacity 60 Litres


No oil Required





AVGAS 100LL & AVGAS 100/130


Leaded & Unleaded MOGAS 95 Octane Ron MIN








Baggage Compartment limit to be displayed in compartment





5Kg MAX





Max 65mm dia to fit inside oil hatch





Oil type Placard located on inside of oil level access hatch





Xair Hawk G - ????





Never Exceed Speed		125 Mph IAS


Manoeuvring Speed			100 Mph IAS


Stall Speed				  35 Mph IAS


Best Climb speed			  65 Mph IAS


Best Glide speed			  60 Mph IAS


Bank angle limits			  +/- 600


Normal Acceleration Limits	  	  +4/-2g





Empty Weight			265 Kg


Empty CG Position			280mm Fwd Datum


Max Take off Weight		450 Kg


Minimum Cockpit Weight		  55 Kg


Max Cockpit Weight			120 Kg each seat





Aerobatics & Deliberate Spinning Prohibited





Sample Cockpit Placard each aircraft will have it’s own specific after Flight Testing and Calibration.





Fuel Load Limitations





Aircraft empty weight 265Kg





Cockpit Load	Fuel Quantity





140 Kg		60 L Full Fuel


148 Kg		50 L	


156 Kg		40 L


163 Kg		30 L


170 Kg		20 L		





Example Based on an Aircraft empty weight of 265Kg will vary on aircraft.








